March 28, 2011

Resonant Frequency Doubler, model FD-SF-07

Unique FD-SF-07 doubler is the most efficient resonant frequency doubler for CW single-frequency lasers in the present market.

General description

- Resonant rigid cavity for high efficiency doubling of cw radiation, power-enhancement factor up to 130.

- Fast two-stage system of locking the cavity to the frequency of input radiation (Hansch-Couillaud polarization method).

Wavelength range, conversion efficiency

- Fundamental mirror set covering approximately 900-950 nm.

- Nonlinear crystal (LBO) for wavelength 922 nm fundamental.

- Conversion efficiency: > 20% at 1W input at 922 nm.

- Stable operation for fundamental pump power <2W.

Design features

- Crystal mount with temperature stabilization, lateral translation and phase matching adjustment.

- Enclosed cavity with nitrogen purge port, express-access to the alignment of the non-linear crystal through a special easily removable lid.

- Electronics module with user-adjustable servo control parameters, separate control photo-detector included into the standard set of the electronic unit.

- Automatic relock electronics.

- Input coupling optics, harmonic separation optics, and output beam-shaping optics (cylindrical lens).

Specifications:

Resonant Frequency Doubler, model “FD-SF-07”:

Wavelength range: 	      922 +/-5 nm

Conversion efficiency:     > 20% at 1W input at 920 nm.

Automatic relock electronics, temperature stabilization of crystal, input coupling optics, harmonic separation optics, and output beam-shaping optics.

Price of the FD-SF-07 doubler with above mentioned specifications, including delivery, installation, teaching and 12-months warranty period:

47,950 USD.

Time of delivery: 12 weeks ARO.